Federal Aviation Administration – William J. Hughes Technical Center

National Airport Pavement Test Facility (NAPTF)

Contract No: DTFACT-15-D-00007

TO 003 Nondestructive Airport Pavement Testing Support

5.1.63 Continuation of NAPTF Construction Cycle Support – CC9 (4.1.10.19a)

CC9 NDT Survey Notes

Device: GPR – Cart

Survey: Weekly Trafficking 20210510

Date: 05/27/2021

Operators: Christopher Mazzotta

Folder Name: 20210527 GPR NDT Van CC9 Weekly Raw Data

File Format: DZT

General Notes

- Data collected along the ten (10) Standard Transverse Test Lines at Stations.

- Cart is aligned with white transverse line and direction of travel is from Offset -26.66 ft. to +26.66 ft. (North to South).

- Survey starts and stops with the antenna centered on the yellow longitudinal edge of test area lines.

- Data collected with 900 MHz and 2.6 GHz ground-coupled antennas.

- Begin with air-coupled antenna centered on transverse blue line at Station 0+00.

- Calibration of DMI performed before data collection with each antenna and value recorded.

Settings

Parameter				900 MHz Antenna		2.6 GHz Antenna

Transmit Rate				100 kHz			100 KHz

Rate (Scans/Second)			100 (default)			100 (default)

Samples/Scan				512				512

Bits/Sample				16				16

Range					15 ns				8 ns

Scans/Unit				30 scans/ft.			90 scans/ft.

Gain Points				3				2

Dielectric				4.00 (default)			4.00 (default)

Vertical Low Pass – LPFIR		0				5000 MHz

Vertical High Pass – HPFIR		0				400 MHz

Vertical Low Pass – LPIIR		2500 MHz			0

Vertical High Pass – HPIIR		225 MHz			10 MHz

DMI Calibration

Distance				900 MHz Antenna		2.6 GHz Antenna

10.00 ft.				-494.2				-494.3

Data Collection File Numbers

Location				900 MHz Antenna		2.6 GHz Antenna

Station 0+15 LFS-1				118				128

Station 0+30 LFS-1				119				129

Station 0+75 LFS-2				120				130

Station 0+90 LFS-2				121				131

Station 1+35 LFC-3				122				132

Station 1+50 LFC-3				123				133

Station 1+95 LFC-4				124				134

Station 2+10 LFC-4				125				135

Station 2+55 LFC-5				126				136

Station 2+70 LFC-5				127				137

Survey Notes

- 900 MHz and 2.6 GHz data does not seem as defined as previous data collections when viewed on the SIR-3000 display. Will require troubleshooting with GSSI.