Federal Aviation Administration – William J. Hughes Technical Center

National Airport Pavement Test Facility (NAPTF)

Contract No: DTFACT-15-D-00007

TO 003 Nondestructive Airport Pavement Testing Support

5.1.63 Continuation of NAPTF Construction Cycle Support – CC9 (4.1.10.19a)

CC9 NDT Survey Notes

Device: SurPro

Survey: Trafficking 20220418

Date: 4/28/2022

Operators: Sean Cullen

Folder Name: 20220428 SurPro CC9 Traffic Raw Data

File Formats: E01, ERD, P01, PFL

Time: NA

General Notes

- A closed loop calibration line was collected before collecting data.

- Time stamps noted here are from the device, not actual.

- Each survey has 4 files, one each in the E01, ERD, P01, PFL formats.

Calibration Survey Line

Number of Survey Lines and Files: 1

Time: 09:42 PM

File Name: 28AP0900

Type: Closed Loop

Length: 75.5’ (one-way), 151.0’ (total)

Survey Line Details:

- NA

Transverse Survey Lines

Number of Survey Lines and Files: 10

Type: Open Loop

Survey Line Details:

- Survey line centered on transverse white test line (Standard Transverse Test Line).

- Starting position is with kickstand aligned with outer edge of longitudinal yellow line marking northern edge of test area.

- Ending position is with kickstand aligned with outer edge of longitudinal yellow line marking southern edge of test area.

- Direction of travel is from North to South.

Location			Time				File Name

Station 0+15			09:01 AM			28AP0901

Station 0+30			09:03 AM			28AP0903

Station 0+75			09:04 AM			28AP0904

Station 0+90			09:05 AM			28AP0905

Station 1+35			09:06 AM			28AP0906

Station 1+50			09:07 AM			28AP0907

Station 1+95			09:08 AM			28AP0908

Station 2+10			09:10 AM			28AP0910

Station 2+55			09:12 AM			28AP0912

Station 2+70			09:15 AM			28AP0915

Longitudinal Survey Lines

Number of Survey Lines and Files: 2

Type: Closed Loop

Survey Line Details:

- Survey lines aligned with offsets -15.00’ and +15.00’ (centerlines of North and South traffic patterns, respectively) using the centers of the white painted circles marking the Standard Test Locations at offsets -15.00’ and +15.00’ for reference.

- Starting position is with kickstand aligned with outer edge of transverse blue line marking the western edge of test area at Station 0+00.

- Ending position is with kickstand aligned with outer edge of transverse blue line marking the eastern edge of test area at Station 2+85.

- Direction of travel is from West to East.

Location			Time				File Name

Offset -15.00’ (North)		09:20 AM			28AP0920

Offset +15.00’ (South)	09:25 AM			28AP0925

Survey Notes

- Dataset 20220418 includes 8 days of trafficking over 2 weeks (4/18/2022 – 4/27/2022).